Characteristics

| Characteristics | Unit | Minimum | Typical | Maximum |
| --- | --- | --- | --- | --- |
| Viewing angle | degrees |  | 120 |  |
| DC forward voltage | V | 42 | 45 | 48 |
| DC forward current | mA |  | 6700 |  |

Flux Characteristics

| Color |  |  | Warm White | Cool White |
| --- | --- | --- | --- | --- |
| Base Number |  |  | R1130G2W30 | R1130G2C55 |
| Luminous Flux Lumen |  |  | 23100 | 27000 |
| Luminous Efficiency (Lm/W) |  |  | 77 | 90 |
| Ra |  |  | 72 | 73 |
| CIE X |  |  | 0.4422 | 0.3324 |
| CIE Y |  |  | 0.4365 | 0.3412 |
| Color Temperature (CCT) | Min |  | 2851 | 5251 |
|  | Typical |  | 3000 | 5500 |
|  | Max |  | 3250 | 5750 |

Note:

- Parts are tested at 25°C.

- Flux measurements have ± 7% tolerance.